Digital Watchdog is a leading manufacturer of security and surveillance solutions, offering stunning image quality, advanced hardware capabilities, reliable customer support and the lowest total cost of deployment to the analog & IP megapixel surveillance markets. Located in Tampa, FL with manufacturing facilities in Seoul, Korea, Digital Watchdog is committed to delivering powerful security solutions to its customers worldwide.

For additional information, contact:

- Digital Watchdog, Inc.

- 16220 Bloomfield Avenue. Cerritos,

- California USA 90703 USA

- Phone: (866) 446-3595

- Web: www.digital-watchdog.com

- E-mail:  dw-tech@digital-watchdog.com

MEGApix® 2.1MP/1080P 30X PTZ IP CAMERA

- DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

2820 00 	Electronic Surveillance

28 23 00 	Video Surveillance

28 23 29	Video Surveillance Remote Devices and Sensors

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented in colored text.

MEGApix® 2.1MP/1080P 30X PTZ IP CAMERA

- GENERAL

  - SUMMARY

    - The section includes a 2.1MP/1080p high definition IP Pan, Tilt, Zoom (PTZ) video camera with vandal-resistant IP-66 rated, IK10 impact-resistant high-speed PTZ housing.

    - Product - A high definition IP PTZ video camera, based on H.265, H.264 and MJPEG compression, capable of quad streaming 30fps at resolutions up to 1920 x 1080, with color in near-total darkness technology and a 4.3 ~ 129mm motorized zoom lens and built-in IR illuminator in an indoor/outdoor high-speed PTZ housing.

    - Related Requirements

      - 27 20 00 Data Communications

      - 28 23 13 Video Surveillance Control and Management Systems

      - 28 23 16 Video Surveillance Monitoring and Supervisory Interfaces

      - 28 23 19 Digital Video Recorders and Analog Recording Devices

      - 28 23 23 Video Surveillance Systems Infrastructure

  - REFERENCES

    - Abbreviations

      - AGC - Automatic Gain Control

      - ARP – Address Resolution Protocol

      - AWB  - Automatic White Balance

      - BLC – Backlight compression

      - DHCP - Dynamic Host Configuration Protocol

      - DNR – Digital Noise Reduction

      - DNS - Domain Name Server

      - DDNS –Dynamic Domain Name Server

      - fps - frames per second

      - FTP - File Transfer Protocol

      - GUI – Graphical User Interface

      - HLC – Highlight Compensation

      - HTTP - Hypertext Transfer Protocol

      - ICMP – Internet Control Message Protocol

      - IGMP - Internet Group Management Protocol

      - IP - Internet Protocol

      - JPEG - Joint Photographic Experts Group

      - MJPEG - Motion JPEG

      - MPEG - Moving Pictures Experts Group

      - NTP - Network Time Protocol

      - PoE - Power over Ethernet

      - PPPoE - Point-to-Point Protocol over Ethernet

      - PTZ – Pan, Tilt, Zoom

      - QoS – Quality of Service

      - RARP – Reverse Address Resolution Protocol

      - RTP - Real-Time Transport Protocol

      - RTSP - Real-Time Streaming Protocol

      - SMTP - Simple Mail Transfer Protocol

      - SNMP – Simple Network Management Protocol

      - TCP - Transmission Control Protocol

      - UDP - User Datagram Protocol

      - UPnP – Universal Plug and Play

      - VMS - Video Management System

      - WDR – Wide Dynamic Range

    - Reference Standards

      - Network

        - IEEE - 802.3 Ethernet Standards

      - Video

        - ISO / IEC 14496 – MPEG-4

          - ISO / IEC 14496–10, MPEG-4 Part 10 ( ITU H.264)

        - ISO / IEC 10918 – JPEG

        - ONVIF–Profile S

      - Emissions

        - FCC-47 CFR Part 15 Class B

      - Environmental

        - ANSI / IEC60529 – Degrees of Protection Provided by Enclosures

        - International Electrotechnical Commission (IEC) – Ingress Protection Rating IP66

        - European standard EN 62262 — equivalent to International Electrotechnical Commission (IEC) – Impact Protection Rating IK10.

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic data sheets

      - Manufacturer’s installation and operation manuals

      - Warranty documentation

  - QUALIFICATIONS

    - The manufacturer shall have a minimum of two years of experience in producing IP video equipment.

    - Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE, AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, and undamaged container with identification labels intact.

    - Store the camera in a temperature environment of -4°F ~ 122°F (20°C ~ 50°C ), protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY AND SUPPORT

    - The manufacturer shall provide a 2-year warranty for the product to be free of defects in material and workmanship.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Digital Watchdog, Inc.

16220 Bloomfield Avenue. Cerritos,

California USA 90703 USA

Phone: (866) 446-3595

Web: www.digital-watchdog.com

E-mail:  dw-tech@digital-watchdog.com

    - Model		DWC-MPTZ230XTW

    - Alternates: None

  - GENERAL DESCRIPTION

    - The high definition IP PTZ Dome Camera (“PTZ dome camera”) shall provide video performance capable of providing selectable resolutions up to 1920 x 1080 pixels at 30 frames per second (fps) with color in near-total darkness technology, contained within an IP-66 rated, IK10 impact-resistant high-speed PTZ dome housing.

    - The PTZ dome camera shall possess the following characteristics:

      - H.265, H.264 and MJPEG compression

      - 2.1MP/1080p

      - Star-Light Plus™ color in near-total darkness technology (0.05 lux)

      - Dual IP video streams (quad streaming)

      - Day/night operation with IR cut filter

      - 30 x optical zoom

      - Integral IR LED illuminator providing 200-foot distance

      - 16 privacy masking regions

      - Integral motion detection

      - True Wide Dynamic Range

      - 3D digital noise reduction

      - Two-way audio

      - Alarm input and output

      - PoE capable

      - Multicast or unicast capable

      - Local storage via Micro SD, SDXC, or SDHC card

      - Dynamic DNS (DDNS) support

      - IP-66 rated

      - IK-10 impact resistant

  - VIDEO

    - Imager

      - Sensor:				1/3“ 2.55M CMOS

      - Minimum illumination

        - Color mode: 		0.05 lux

        - Black & white mode:	0.0 lux

      - Scanning - Progressive

      - Image Control Settings shall be available for:

        - Automatic white balance (AWB)

        - Exposure modes: 		automatic or manual

          - In manual exposure mode, configurable settings for:

            - brightness

            - shutter mode: 	automatic 1/10000

            - digital slow shutter up to 4x to decrease the shutter speed in low light

            - Automatic Gain Control (AGC)

            - Backlight compensation (BLC)

            - WDR Level:		120 dB minimum

        - Day and night settings, to allow configuration for Day (color), Night (Black and White), or Automatic.

        - Frequency

        - Image sharpness

        - Chroma

        - Image mirror or flip

        - Digital noise reduction

        - Privacy zone definition, up to 16 zones

      - Lens:

        - Lens type: 4.3~129mm, F1.6~F4.7, Motorized Zoom PTZ Lens

        - Optical zoom: 30x

        - Field of view (FOV): 65.1°~2.34°

        - IR Distance: 200ft range

        - Double shutter Wide Dynamic Range, 120dB

        - Shutter speed: 	1/8 ~ 1/30,000

    - Video Streams

      - The PTZ dome camera shall support two configurable video streams, each of which may have the following properties:

        - Resolution:

          - H.265: 2.1MP/1080p, 720p, VGA, D1, CIF

          - H.264: 2.1MP/1080p, 720p, VGA, D1, CIF

          - MJPEG: 1280x720, 800x600, VGA, 768x432, D1, CIF

        - Compression type:

          - H.265 (main profile)

          - H.264 (baseline, main, high profile)

          - MJPEG

        - Bit rate:	100 Kbps – 10 Mbps

        - Frame rate:	0 – 30 fps

    - Motion Detection – The PTZ dome camera shall have the ability to detect motion within user-defined areas of the video image.

      - Configuration settings shall be available for sensitivity and dwell time.

    - Storage and Recording

      - The PTZ dome control shall have onboard SD card storage.

        - Card Type:	Micro SD, SDHC and SDXC Class 10

      - Local recording on the SD card shall commence upon loss of network connectivity, based on a pre-programmed schedule.

      - The local SD storage shall have the ability to be backed up to alternate media without the removal of the SD card from the camera.

    - ONVIF – Video streams shall be capable of supporting ONVIF protocol, profile S.

    - Pan Tilt

      - Pan Range:	360o, endless

      - Tilt Range:	5 o ~ 90o

      - Presets:	255

      - Tours:		up to 10

      - Speed:		360o per second

    - Other

      - Single Images - The PTZ dome camera shall support jpg file image screenshot and export.

      - Discovery - The manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.

      - Emergency alarm – The PTZ dome camera shall allow the sending of a video clip of configurable duration to up to five server locations.

      - Access- The PTZ dome camera shall permit up to five users to simultaneously access the camera.

      - Alarm/Sensors – The PTZ dome camera shall have three alarm/sensor inputs and one alarm/control output.

  - AUDIO

    - The PTZ dome camera shall provide one line-level input and one line-level output.

    - Compression and Sampling Rate

      - G.711 (8 kHz)

  - NETWORK

    - Connectivity: 10/100 BASE-T Ethernet with RJ-45 connector

    - Protocols supported

      - Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

      - Configuration: Dynamic Host Configuration Protocol (DHCP)

      - Web services: Hypertext Transfer Protocol (HTTP)

      - Network services: Universal Plug and Play (UPnP)

      - Media: Real-Time Transport Protocol (RTP), Real-Time Streaming Protocol (RTSP)

      - Notifications: File Transfer Protocol (FTP)

    - Flow Control- The PTZ dome camera shall support TCP and Layer 3 QoS protocols to control network congestion.

    - DDNS – The PTZ dome camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

    - Security

      - The PTZ dome camera shall support IP address filtering whereby users can enter a list of allowed or blocked IP addresses for viewing video and configuring camera settings

      - The PTZ dome camera shall provide three levels of user access with password protection.

  - ADDITIONAL FEATURES

    - Auxiliary Inputs and Outputs

      - Input: Alarm sensor (contact closure)

      - Output: Relay

    - System Information

      - The system settings of the PTZ dome camera shall be exportable as a separate file.

      - The PTZ dome camera shall maintain an access log of the system and motion-triggered events.

        - The log shall be exportable to an Excel spreadsheet file.

  - CAMERA SOFTWARE

    - The PTZ dome camera shall have a built-in web server that supports browser-based configuration using Internet Explorer, Google Chrome, Mozilla Firefox, and Apple Safari.

    - The software GUI shall allow access to camera information and all primary software functions, including:

      - Camera network address

      - Configuration

      - Stream control

      - Screen snapshot, print and export

      - Start and stop recording

    - The Manufacturer shall offer a video viewer and configuration to implement the following actions:

      - Camera discovery

      - Configuration

        - video parameters

        - events and notifications

          - motion-related

          - emergency

        - camera network parameters

        - SD card storage recording management

        - image capture, export, and print

      - Viewer - view video streams through the web browser

      - Image print and export

      - Instant record and playback

      - Alerts

        - e-mail setup

        - define web addresses for notifications

      - System

        - firmware upgrade

        - reset to factory default

        - set date, time, and NTP server synchronization

        - user access control

        - view and export camera settings

        - view system logs

  - ELECTRICAL

    - Power

      - Sources

        - 24 VDC

        - 24 VAC

        - PoE IEEE 802.3bt class5

      - Power Consumption:

        - DC24V: 40W, 1666mA

        - AC24V: 35W, 1485mA

        - PoE: 42W, 1750mA

      - Ethernet: 			RJ-45 connector

      - External power:			2-conductor power to the terminal block

      - Audio:				individual 2 wire connections for in and out

      - Alarm In and Out:		up to 4 wire connection to the terminal block

  - MECHANICAL AND ENVIRONMENTAL

    - Housing Material: 		Aluminum housing, polycarbonate dome

    - Configuration:		PTZ dome

    - Dimensions (D x H): 	5.9" x 9.13" (150 x 232mm)

    - Weight: 			6.17 lbs (2.8 Kg)

    - Temperature:

      - Operating:		-4°F ~ 122°F (20°C ~ 50°C )

    - Humidity:			10 - 90%, non-condensing

    - Environmental Rating: 	IP-66

    - IK Rating: 			IK10

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s). Network configuration shall be tested and qualified by the Contractor before camera installation.

  - INSTALLATION

    - Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment

      - A report indicating successful test results shall be produced.

  - STORAGE

    - The PTZ dome camera hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.

END OF SECTION